IrrigationController
Consolidated Software & Hardware Specification

Version 1.0 — Implementation Baseline

| Item | Value |
| --- | --- |
| Purpose | Wi-Fi irrigation controller with local web interface and programmable irrigation |
| MCU | ESP32-WROOM-32E, LCSC C701342 |
| PCB status | Hardware design frozen; PCB/PCBA prepared for JLCPCB |
| Role | Single implementation reference for firmware development and Codex |

# 1. Scope and status

This document consolidates the previously separate software specifications into one implementation-oriented reference. The original documents remain archival/source material; firmware implementation should use this document as the primary baseline. The architecture remains modular, deterministic, testable, event-driven and hardware-independent above the HAL.

The PCB design has been frozen and prepared for JLCPCB assembly. Hardware-specific assumptions that still require physical-board verification are explicitly identified rather than guessed.

# 2. Final hardware platform

| Block | Final hardware | Software consequence |
| --- | --- | --- |
| MCU | ESP32-WROOM-32E (U301), LCSC C701342 | Main controller and Wi-Fi MCU |
| USB | CP2102N-Axx-xQFN24 (U302) | USB/serial development and service |
| I/O expander | MCP23017T-E/SS (U401) | Additional digital I/O |
| Output driver | ULN2803A (U501) and relay drive circuitry | Hidden behind Output Driver/HAL |
| Actuators | 8 × Omron G5LE-family 5 V relays K601–K608 | Eight logical output channels |
| Field output | J702, 8 positions; relay contacts switch 24 VAC valve circuit | Logical zones map to physical outputs |
| Master Valve | Remote valve controlled by Shelly 1 Gen 3 over Wi-Fi/LAN | Controlled by Master Valve Service through a dedicated Shelly client/driver; independent of local PCB relay outputs |
| Power | IRM-10-5 (U201), AP2112K-3.3 (U202) | 5 V / 3.3 V logic rails |
| Inputs | J703 INPUT1, J704 INPUT2 | Two external digital inputs |
| Controls | SW301 RESET, SW302 BOOT | Reset/recovery |
| Status | D301 status; D201/D202 power; D601–D608 relay LEDs | Diagnostics/status |

Application code must not depend on raw ESP32 GPIO numbers, MCP23017 registers, ULN2803 details or relay implementation. These belong to the HAL/output-driver layer.

# 3. Hardware-aligned decisions

- The ESP32-WROOM-32E is the primary controller.

- The eight relay channels are the physical actuator resources. Higher layers use logical Output/Zone IDs.

- The eight irrigation zones are controlled locally by the PCB relay outputs. The Master Valve is a remote actuator controlled by a Shelly 1 Gen 3 over the network and is therefore not part of the PCB relay/output mapping.

- The Master Valve Service communicates with the Shelly through a dedicated Shelly client/driver abstraction. Communication failures, timeouts and unreachable-device conditions must be handled explicitly and must prevent unsafe irrigation sequencing.

- The exact MCP23017 channel mapping and exact logical-to-physical relay mapping must be verified on the first assembled PCB.

- All valve outputs must default to a safe OFF state at boot/reset/fault unless a deliberate, verified recovery policy says otherwise.

- No blocking delay() is permitted in the scheduler/control path.

# 4. Software architecture

| Layer | Responsibility |
| --- | --- |
| Web UI | Presentation only |
| REST API / WebSocket | Transport; no irrigation business logic |
| Application services | Scheduler, Program, Zone, Master Valve, Configuration, Wi-Fi, OTA, Logger |
| Domain logic | Rules, validation, sequencing and state transitions |
| Command/Event infrastructure | Command Bus and Event Bus |
| State Store | Single source of truth for current runtime state |
| HAL / Drivers | ESP32, GPIO, MCP23017, relay/output hardware, Wi-Fi, storage and time |

Core processing pipeline:

Request → Command → Validation → Handler → Service → State Store → Event → Subscribers

# 5. Core modules

| Module | Responsibilities |
| --- | --- |
| Scheduler | Evaluate time/schedules and trigger programs; never drives outputs directly |
| Program Service | Programs, steps, sequencing, start/pause/resume/stop |
| Zone Service | Open/close zone, duration, enforce one active zone |
| Master Valve Service | Open/close the remote master valve through the Shelly client, manage delays and timeouts, and handle communication failures |
| Shelly Client | Network communication with the remote Shelly 1 Gen 3; provides an abstract interface for switching and obtaining the relevant device status |
| Output Driver | Translate logical outputs to actual ESP32/MCP23017/ULN2803/relay hardware |
| Configuration Manager | Persistent JSON; load/save/backup/restore |
| Wi-Fi Manager | Connect/reconnect, AP mode, NTP |
| REST Controller | Queries and Command transport |
| WebSocket | Real-time Event/state delivery |
| OTA Manager | Download, verify, install, rollback/recovery |
| Logger | DEBUG/INFO/WARNING/ERROR/CRITICAL |
| State Store | Current state only; no history or persistent configuration |

# 6. Messaging model

- All state changes are initiated through immutable Commands.

- Events are immutable facts that have already happened; they do not execute business logic.

- Queries are read-only.

- The State Store is the single source of truth.

- Commands never directly generate GPIO, HTTP or hardware actions.

- Each Command has a Handler; handlers validate syntax, authorization and current state before invoking a Service.

- Events may have multiple subscribers such as Logger and WebSocket.

- Commands and Events are versioned.

- Commands should be idempotent where practical.

| Command group | Initial catalog |
| --- | --- |
| Zone | OpenZoneCommand, CloseZoneCommand, EnableZoneCommand, DisableZoneCommand, ResetFaultCommand |
| Program | StartProgramCommand, PauseProgramCommand, ResumeProgramCommand, StopProgramCommand, EnableProgramCommand, DisableProgramCommand |
| System | RebootControllerCommand, StartOTACommand, SaveConfigurationCommand, RestoreConfigurationCommand |
| Network/User | ConnectWiFiCommand, DisconnectWiFiCommand, LoginCommand, LogoutCommand |

# 7. Canonical data model

Canonical data is shared by configuration, State Store, Commands, Events, Queries, Results, REST, WebSocket and persistence. IDs are stable strings; GPIO numbers are never domain IDs. Timestamps are ISO-8601 UTC; durations are milliseconds.

| Entity | Core fields |
| --- | --- |
| Zone | id, name, enabled, outputId, defaultDuration, description |
| Output | id, driver, channel, enabled |
| Step | id, type, target, duration, parameters |
| Program | id, name, enabled, scheduleId, ordered steps |
| Schedule | id, type, days, startTime |
| Master Valve | enabled, Shelly endpoint/device reference, confirmation policy, open/close delays/timeouts |
| User | id, role, enabled |
| State | system, zones, programs, masterValve, network, alarms |
| Zone State | zoneId, state, remainingTime |
| Program State | programId, state, currentStep |
| Network State | connected, RSSI, IP |

# 8. State machines

Global states: BOOT, READY, RUNNING, MANUAL, OTA, MAINTENANCE, ERROR.

- BOOT → READY after successful initialization.

- READY → RUNNING on StartProgramCommand.

- READY → MANUAL when manual operation starts.

- RUNNING → READY when no program remains active or a program is stopped/completed.

- MANUAL → READY after manual operation exits.

- * → OTA on authorized StartOTACommand.

- OTA → READY after successful update/reboot.

- * → ERROR on a critical HardwareFaultEvent.

- ERROR → BOOT on RebootControllerCommand.

Program states: IDLE, STARTING, RUNNING, PAUSED, STOPPING, COMPLETED, ERROR.

Zone states: DISABLED, IDLE, OPENING, OPEN, CLOSING, FAULT.

Master Valve states: CLOSED, OPENING, OPEN, CLOSING, FAULT.

Wi-Fi states: DISCONNECTED, CONNECTING, CONNECTED, AP_MODE, ERROR.

OTA states: IDLE, DOWNLOADING, VERIFYING, INSTALLING, REBOOTING, ERROR.

# 9. Irrigation execution

- Scheduler detects a valid schedule.

- StartProgramCommand is generated/processed.

- Program Service validates the program and starts it.

- Master Valve Service commands the remote Shelly 1 Gen 3 to open the master valve. Irrigation zones are started only after the master-valve opening operation has succeeded according to the configured confirmation policy.

- Zone Service opens one zone through the logical Output Driver.

- The zone runs for its configured duration without blocking the rest of the firmware.

- The zone closes and the next zone is started.

- After the final zone, the Master Valve Service commands the Shelly 1 Gen 3 to close the master valve after the configured delay.

- ProgramCompletedEvent is emitted and the system returns to READY when no other program is active.

The one-active-zone rule is mandatory. Logical zones are mapped to the eight physical relay outputs through configuration.

# 10. REST API

Base URL: /api/v1. JSON over UTF-8. Timestamps: ISO-8601 UTC.

| Method | Endpoint | Purpose |
| --- | --- | --- |
| GET | /api/v1/state | Entire State Store |
| GET | /api/v1/zones and /api/v1/zones/{id} | Zones |
| GET | /api/v1/programs and /api/v1/programs/{id} | Programs |
| GET | /api/v1/scheduler | Scheduler |
| GET | /api/v1/network | Network |
| GET | /api/v1/master-valve | Master valve |
| GET | /api/v1/system | System |
| GET | /api/v1/logs | Logs |
| POST | /api/v1/commands | Transport any supported Command |

REST performs authentication/authorization and interface validation, but never implements irrigation business logic or accesses hardware directly.

# 11. WebSocket

WebSocket is the real-time notification channel. It subscribes to the Event Bus and sends relevant Events/state changes to clients using the canonical data model. It contains no domain logic and cannot directly manipulate outputs.

# 12. Persistence, time and security

- Persistent configuration is JSON and includes system, zones, outputs, programs, schedules, users, network and masterValve.

- Runtime State Store is separate from persistent configuration and logs.

- Passwords/secrets are never stored or logged in plaintext; password storage uses hashing.

- NTP is used when network time is available; canonical timestamps remain UTC.

- Scheduler timing uses millis(), FreeRTOS timers or equivalent non-blocking mechanisms.

- OTA images must be verified before installation.

- Roles are Administrator, Operator and Guest.

# 13. FreeRTOS/task model

| Execution context | Role |
| --- | --- |
| Command Task | Receive/dispatch Commands |
| Scheduler Task | Evaluate schedules/time triggers |
| IO Task | Hardware-facing logical I/O work |
| Network Task | Wi-Fi/network processing |
| Logger Task | Asynchronous logging |
| OTA Task | Firmware update workflow |
| Web/API context | HTTP/WebSocket transport without blocking domain operations |

Queues, event groups and timers are preferred for coordination. Long blocking operations must not stall irrigation control.

# 14. Error handling

| Result | Meaning |
| --- | --- |
| OK / Success | Operation completed |
| Rejected | Command valid but not executable |
| Forbidden | Authorization failure |
| InvalidState | Current state does not permit operation |
| NotFound | Referenced entity does not exist |
| Timeout | Operation exceeded its limit |
| InternalError | Unexpected internal error |

REST mapping follows the existing specification: 200/201 success, 400 malformed request, 401 unauthenticated, 403 unauthorized, 404 not found, 409 incompatible state, 422 valid but not executable, 500 internal error.

# 15. Logging and testing

- Log important program, master-valve, zone, network, configuration, OTA and fault events.

- Levels: DEBUG, INFO, WARNING, ERROR, CRITICAL.

- Domain modules must be unit-testable without an ESP32.

- State-machine transitions require normal, invalid, timeout and fault tests.

- REST/WebSocket tests are independent of domain logic.

- After the first assembled PCB arrives, perform hardware-in-the-loop tests before declaring the hardware support complete.

# 16. First-board hardware verification checklist

- Verify exact MCP23017 channel assignment.

- Verify exact logical-to-physical relay mapping K601–K608.

- Verify relay active polarity and ULN2803A drive behavior.

- Verify all eight 24 VAC outputs under the intended field load.

- Verify INPUT1 and INPUT2 electrical behavior.

- Verify RESET/BOOT behavior.

- Verify power-up/reset/watchdog behavior leaves valve outputs safely OFF.

- Verify Wi-Fi loss does not create unsafe actuator behavior.

- Verify OTA/reboot behavior leaves outputs in a safe state.

# 17. Recommended firmware repository structure

firmware/
  app/
    services/ state/ commands/ events/ scheduler/
  domain/
    zones/ programs/ master_valve/
  infrastructure/
    persistence/ logging/ network/ ota/
  hal/
    esp32/ gpio/ mcp23017/ outputs/
  interfaces/
    rest/ websocket/
  main/
tests/
docs/
AGENTS.md

# 18. Implementation order

- Create repository skeleton, build system and AGENTS.md.

- Implement HAL and verify the finalized PCB output/input mapping.

- Implement State Store and canonical structures.

- Implement Command/Event infrastructure and Result types.

- Implement Zone Service and Output Driver.

- Implement Master Valve Service.

- Implement Program Service and program state machine.

- Implement Scheduler and time synchronization.

- Implement persistent configuration.

- Implement Wi-Fi Manager.

- Implement REST API and authentication.

- Implement WebSocket event delivery.

- Implement OTA.

- Add unit, integration and hardware-in-the-loop tests.

# 19. Important reconciliation with the original drafts

The consolidated baseline retains the Command Bus + Event Bus + State Store architecture and its modular/HAL principles. The finalized hardware architecture distinguishes between the eight local irrigation-zone outputs provided by the PCB and the remote Master Valve. The eight irrigation zones are controlled through the PCB Output Driver, while the Master Valve is a remote actuator controlled through a Shelly 1 Gen 3 over the network. Master Valve communication is isolated behind a dedicated Shelly client/driver interface. Exact MCP23017 and relay mappings that cannot be established from the design documentation remain first-board verification items rather than assumptions..

# 20. Source documents

- Functional Specifications / requirements

- 01 - Software Architecture Document (SAD) v1.0

- 02 - Domain Model v1.0

- 03 - Data Dictionary v1.0

- 04 - Protocol Design Document (PDD) v1.0

- 05 - Canonical Data Model (CDM) v1.0

- 06 - State Machine Specification v1.0

- 07 - System State Machine v1.0

- 08 - Task Model v1.0

- 09 - REST API Specification v1.0

- 10 - WebSocket Protocol Specification v1.0

This document is the implementation baseline; the source documents remain useful for detailed traceability and historical context.